Faceless FRS

Multi-Modal Biometric Suspect Identification

Face · Gait · Appearance, fused under human confirmation

Engineering and Evaluation Report

Extending Prakash et al. (2023), arXiv:2303.13814

37 commits · 458 automated tests · 14,495 lines of source

# Contents

Contents	2

1. Executive summary	4

1.1 Headline results	4

1.2 What changed during this engineering cycle	4

2. System architecture	5

2.1 Pipeline stages	5

2.2 Why three signals	5

2.3 Fusion, and what it is not	5

2.4 Human confirmation	5

3. Results	7

3.1 Enrolled reference (case-0003)	7

3.2 Identification results — six test clips	7

Separation from bystanders	7

3.3 The qualification: what carried each decision	7

4. Engineering findings	9

4.1 Appearance was vetoing correct face matches	9

4.2 A covered face scored higher than an uncovered one	9

Validation, and its limits	10

4.3 Gait had never been given a walking video	10

4.4 Appearance exhausted the GPU during enrolment	10

4.5 Enrolment destroyed the frame timestamps gait needs	10

4.6 Silhouette clipping measured the wrong edge	11

4.7 An attempted fix that was measured and reverted	11

5. Evaluation methodology	12

5.1 Calibration measurement	12

5.2 Camera placement, measured	12

5.3 Enrolment angles	12

6. Safeguards and data handling	14

7. Technology	14

8. Limitations	14

Appearance does not survive a change of clothes	15

Gait is enrolled but not yet operational	15

Gait may not survive live scanning	15

Profile and heavily disguised subjects	15

Single template per person	15

Small evaluation set	15

9. Future work	15

Appendix A. Development history	17

Appendix B. Key configuration	18

# 1. Executive summary

Faceless FRS identifies people in CCTV footage by fusing three independent biometric signals — the face, the way a person walks, and their overall appearance — and presents every result to a human for confirmation. Nothing is ever acted on automatically.

The system works. On the most recent test set it correctly identified the enrolled subject in six of six clips, including clips where he wore a surgical mask and a full-face motorcycle helmet, and it separated him from bystanders walking through the same frame by a margin of 0.81 to 0.91 on a 0-1 scale.

That headline needs one qualification, stated here rather than buried: five of those six identifications rested on appearance — clothing and build — rather than on the face. Appearance is a strong signal within a single session and a weak one across days. The system reports this itself on every match, which is the part of the design worth defending most.

## 1.1 Headline results

| Question | Answer |
| --- | --- |
| Correct identification rate, latest test set | 6 of 6 clips |
| Margin over other people in frame | 0.810 – 0.909 |
| Highest false positive from a bystander | 0.079 (threshold is 0.550) |
| Identification with a face mask | Succeeded (0.860, 0.907) |
| Identification with a full-face helmet | Succeeded (0.984, 1.000) — appearance only |
| Face-driven identification | 0.778, with face carrying 63% of the decision |
| Modalities enrolled for the test subject | All three: face, gait, appearance |
| Automated tests | 458 passing |

## 1.2 What changed during this engineering cycle

Six substantive defects were found and fixed. Each was diagnosed by measurement rather than inspection, and each carries a regression test. Two of them had been silently preventing a whole modality from ever working.

| # | Defect | Effect before fix |
| --- | --- | --- |
| 1 | Fusion weighted by crop quality alone | A correct face match at 0.74 was dragged to 0.30 by an appearance score that calibrated to exactly zero yet held 58% of the vote |
| 2 | Face quality ignored occlusion | A masked face scored HIGHER than an unobstructed one (0.642 vs 0.596) |
| 3 | Gait was never given a walking video | Gait had never enrolled for anyone, in the project’s entire history |
| 4 | Appearance batch was unbounded | CUDA out-of-memory mid-enrolment: a 3.41 GiB allocation on a 4 GB card |
| 5 | Enrolment destroyed frame timestamps | Gait saw 1 Hz instead of 30 Hz and refused every upload |
| 6 | Silhouette clipping measured the wrong edge | Fully visible bodies flagged as cut off 89% of the time |

# 2. System architecture

Footage enters a detection and tracking pipeline, each tracked person accumulates a bounded buffer of image crops, three embedding branches read that buffer independently, and a fusion stage combines whatever they produced into one comparable score against the watchlist. Every score above threshold becomes a pending decision for a human.

## 2.1 Pipeline stages

| Stage | Component | Role |
| --- | --- | --- |
| Detection | YOLOv8n | Locate every person in each frame |
| Tracking | DeepSORT | Give each person a stable identity across frames |
| Buffering | TrackBuffer | Hold a bounded ring of crops per track |
| Face | ArcFace (buffalo_l) | Identity from facial structure |
| Gait | GEI descriptor | Identity from walking rhythm and silhouette |
| Appearance | OSNet (MSMT17) | Identity from build and clothing |
| Fusion | quality_weighted | Combine calibrated scores into one number |
| Review | Console + audit | Human confirms or rejects; everything is logged |

## 2.2 Why three signals

Each modality fails in a different situation, and the failures are largely independent. A face is useless when someone turns away; gait is useless when they are seated; appearance is useless once they change clothes. Fusing them means a failure in one is covered by the others — provided the failing branch steps aside rather than voting confidently. Making that true was the largest engineering theme of this cycle.

| Signal | Strong when | Fails when |
| --- | --- | --- |
| Face | Head is turned toward the camera | Turned past ~65°, or covered |
| Gait | Person is walking, viewed side-on | Seated, standing, or too few steps |
| Appearance | Same session, same clothing | Different day or different outfit |

## 2.3 Fusion, and what it is not

A common misconception is that the system tries the face first and falls back to gait, then appearance. It does not. That strategy exists in the codebase as SingleBestFusion and is retained only as a comparison baseline. The active strategy is quality_weighted, which fuses every available modality simultaneously, each weighted by:

- Quality — how good a look this sighting got

- Trust — how stale the stored reference is (appearance decays with a 3-day half-life)

- Separation — how much discriminating room the modality has at all

The third factor was added during this cycle and is described in section 4.1. Raw similarities are first mapped onto a common 0-1 scale by per-modality calibration anchors, because a face similarity of 0.60 and an appearance similarity of 0.60 do not mean remotely the same thing.

## 2.4 Human confirmation

No match causes an action. Every candidate above threshold becomes a pending decision carrying the enrolment photograph beside the frame it was found in, the per-modality breakdown, and the weight each modality carried. A named operator confirms or rejects. The decision, the reasoning and the operator are written to an append-only audit trail.

Design consequence worth noting. Because the review console shows the weight breakdown, a match resting entirely on clothing is visibly different from one resting on a clear face — even though both may show a high score. Section 3.3 shows why that distinction is not cosmetic.

# 3. Results

All figures below are measured, not estimated. Each was produced by running the shipping code paths against real footage of a consenting subject on the hardware described in section 7.

## 3.1 Enrolled reference (case-0003)

The subject was enrolled from three videos: a slow head turn, a side-on walk, and a turn on the spot. This is the first watchlist entry in the project’s history to carry all three modalities.

| Modality | Quality | Frames used | Model |
| --- | --- | --- | --- |
| Face | 0.599 | 197 | insightface/buffalo_l |
| Gait | 0.713 | 600 | gei/32x22@64x44 |
| Appearance | 0.910 | 1,745 | osnet_x1_0/msmt17 |

## 3.2 Identification results — six test clips

Footage shot with the camera mounted high and pointed along a walkway, so subjects approach the lens. Three clips contain other people walking alongside the subject. The decision threshold is 0.550.

| Clip | Score | Face sim. | Appear. sim. | Runner-up | Margin | Result |
| --- | --- | --- | --- | --- | --- | --- |
| test1 | 0.778 | 0.679 | 0.865 | — | — | Matched |
| test2 | 0.889 | no face | 0.864 | 0.079 | 0.810 | Matched |
| test3 | 0.860 | no face | 0.859 | 0.037 | 0.823 | Matched |
| test4 | 0.984 | no face | 0.878 | 0.075 | 0.909 | Matched |
| test5 | 1.000 | no face | 0.887 | — | — | Matched |
| test6 | 0.907 | no face | 0.867 | — | — | Matched |

Subject conditions: test3 and test6 with a surgical mask; test4 and test5 with a full-face motorcycle helmet; test2, test3 and test4 with other people in frame.

### Separation from bystanders

The margin is the number that matters for a suspect-identification system. A high score means little if an innocent bystander scores nearly as high. Raw appearance similarities, subject versus others in the same frame:

subject     0.859   0.864   0.867   0.878   0.887

bystanders  0.412   0.473   0.482   0.519   0.585

Clean separation, with no bystander approaching the 0.550 threshold. The highest false candidate across all six clips scored 0.079.

## 3.3 The qualification: what carried each decision

Reading only the score column would be a mistake. The weight breakdown shows that five of the six identifications rested entirely on appearance:

| Clip | Score | Face contribution | Appearance contribution | Subject’s face |
| --- | --- | --- | --- | --- |
| test1 | 0.778 | 63% | 37% | Visible |
| test2 | 0.889 | 0% | 100% | Turned away |
| test3 | 0.860 | 0% | 100% | Masked |
| test4 | 0.984 | 0% | 100% | Helmet |
| test5 | 1.000 | 0% | 100% | Helmet |
| test6 | 0.907 | 0% | 100% | Masked |

A score of 1.000 for a person in a full-face helmet does not mean the system is certain who they are. It means their outfit is identical to the outfit they were enrolled in. The subject wore the same checked shirt in all six test clips and in the enrolment video, so appearance had the easiest possible task.

The evidence that this matters comes from an earlier measurement on footage where the enrolment outfit and the test outfit differed:

same person, different clothing   0.493 - 0.814

different people                  0.516 - 0.710      <- overlapping

Across a change of clothes, appearance cannot reliably tell the subject from a stranger. The 0.86-0.89 figures in section 3.2 are a same-session result and should be reported as such. test1 — the 0.778 with face carrying 63% — is the only result that would survive a change of outfit, and is therefore the most important row in the table.

The review console states this on every affected match: "100% of this rests on appearance — build and clothing. That is the weakest of the three signals and it goes stale as people change clothes. Look at the faces, not the score."

# 4. Engineering findings

Each subsection states the defect, the measurement that exposed it, the fix, and the measurement that confirms it. Where an earlier conclusion was overturned by later evidence, that is recorded rather than quietly corrected.

## 4.1 Appearance was vetoing correct face matches

A scan found nobody, although the subject was enrolled and on screen throughout. The face branch had in fact identified him correctly in every scored frame. Fusion then discarded that:

| Modality | Raw similarity | Calibrated | Fusion weight |
| --- | --- | --- | --- |
| Face | 0.7884 | 0.740 | 0.42 |
| Appearance | 0.5632 | 0.000 | 0.58 |

Fused result: 0.42 × 0.74 + 0.58 × 0.000 = 0.347, against a threshold of 0.550. On the same frame the nearest wrong person scored 0.17 on face, so the identification was unambiguous and was thrown away.

Two causes compounded. Appearance calibration anchors declare that a stranger scores 0.726; on real footage everybody scored 0.45-0.59, so every appearance score clipped to exactly zero. And fusion weighted by crop quality, which measures how clear a picture is, not how informative the modality is — so the branch contributing nothing held the majority vote because its crop happened to look good.

Fix. Weight became quality × trust × separation, where separation is the gap between a modality’s stranger and genuine anchors. Face spans 0.92; appearance spans 0.155. A modality with a sixth of the discriminating room no longer outvotes one with six times as much.

| Clip | Before | After | Threshold 0.550 |
| --- | --- | --- | --- |
| test1 | 0.3470 | 0.6695 | Now matches |
| test2 | 0.3134 | 0.5996 | Now matches |

## 4.2 A covered face scored higher than an uncovered one

Face quality was computed as detection confidence × frontality × resolution. None of those notice occlusion. Measured on a real enrolment crop:

| Condition | Face covered | Similarity | Quality (before) | Quality (after) |
| --- | --- | --- | --- | --- |
| No disguise | 0% | 1.0000 | 0.596 | 0.596 |
| Surgical mask | 36% | 0.6277 | 0.642 | 0.263 |
| Sunglasses | 15% | 0.8102 | 0.548 | 0.428 |
| Hood | 47% | 0.8728 | 0.524 | 0.524 |
| Mask + sunglasses | 52% | 0.2480 | 0.610 | refused |

The mask row is the problem in miniature: identity substantially degraded, and the branch grew MORE confident. The mechanism is mechanical — an opaque shape is a clean, high-contrast region, so the detector is surer it found a face, while yaw and pixel count do not move. Since fusion weights by quality, the branch that had stopped working was handed the vote.

Fix. A fourth quality factor: visibility. Following Dhamecha et al. (PLoS ONE, 2014), the face is divided into eight regions located from InsightFace keypoints, each classified covered or clear from an intensity-and-texture feature, and the share of visible face multiplies into quality. Unlike the paper, the result drives quality rather than the embedding, because ArcFace reads a whole face at once and cannot be given one with holes in it.

### Validation, and its limits

The classifier is fitted on synthetic occlusions, so it is necessarily good at recognising synthetic occlusions. Two checks push past that.

- Held out by occluder type — fit without one covering, tested on it. Every flat and textured occluder scored AUC 0.999 or better.

- Transfer to shapes the fit never saw: mask lights up nose and mouth on 9 of 10 faces, sunglasses lights up both eyes on 7 of 10, and clean faces produce zero false alarms.

Known limit, stated in the source. Random noise scores AUC 0.000 — not chance, but confidently backwards, because a noise patch has more local variation than skin. This is also why an unlearned "flatness" rule was rejected: it fails the same way on striped patterns. No claim is made about real cloth masks under real lighting; that needs real disguise footage such as DFW.

## 4.3 Gait had never been given a walking video

This is the finding with the longest reach. Gait had never enrolled successfully for anyone, across every attempt in the project’s history, and three separate diagnoses had blamed the footage. All three were measuring the wrong file.

Enrolment caps each video at 600 observations. The subject’s face video (756 raw frames) and walking video (1,269 raw frames) therefore both capped at exactly 600 — a tie. The selector used a strict greater-than, so the tie was broken by upload order:

face.mp4         indices    0..600     (756 raw frames, capped)

walk.mp4         indices  600..1200    (1269 raw frames, capped)

appearance.mp4   indices 1200..1745

gait_segment: (0, 600)   <- face.mp4

Gait was analysing a video of someone standing still turning their head, and reporting, entirely accurately, "legs barely move; person is not walking".

Fix. Every video’s run is now offered to gait in turn, longest first, keeping the first that yields a signal. Length now decides only the order tried, because length was never the question; whether a clip contains a walk is something only the gait branch can answer.

candidate 0 (face.mp4):  refused, legs barely move

candidate 1 (walk.mp4):  ENROLLED  quality 0.713

periodicity 0.713 (gate 0.55)   cycles 4.05

swing 0.287 (gate 0.12)         area_cv 0.062 (gate 0.10)

A retracted conclusion. An earlier analysis in this project’s notes argued that the silhouette area-stability gate of 0.10 had been calibrated on synthetic data and might be unachievable on real video, having measured 0.145-0.205 across three real walks. That was wrong. A genuine side-on walk measures 0.062, comfortably inside the gate. Every one of those earlier measurements had been taken on the wrong file. The gate was correct; the input was not.

## 4.4 Appearance exhausted the GPU during enrolment

Enrolment failed with a CUDA out-of-memory error after the operator had uploaded everything and waited. The appearance branch stacked every usable observation into a single tensor and pushed it to the GPU. Three 60 fps clips produced roughly 1,800 crops; OSNet’s first convolution over that batch is about 3.4 GB, which matches the reported 3.41 GiB allocation on a 4 GB card.

Fix. The forward pass is chunked at a configurable batch size (default 64). Peak memory is now a function of that setting rather than of how long somebody filmed for. Results are bit-identical — OSNet has no cross-sample dependency — and a test asserts chunked and unchunked outputs agree.

## 4.5 Enrolment destroyed the frame timestamps gait needs

The upload path renumbered observations and assigned the frame index as the timestamp, making consecutive frames appear one second apart. Gait requires at least three samples across a 0.28 s half-cycle; at the resulting 1 Hz it received 0.28 and refused every upload with "frames too far apart to measure cadence". Frame indices are counters and may be renumbered; timestamps are the only record of capture rate. Each clip’s real spacing is now preserved.

## 4.6 Silhouette clipping measured the wrong edge

Gait discounted frames where the body was "cut off", detected by testing whether the silhouette mask touched the top or bottom row of its own crop. But the crop is the detection box, drawn tight around the person, so a fully visible body touches both rows by construction. Measured on a walking clip where not one detection box came within 3 px of the frame edge, the test flagged 89% of frames as clipped, cutting gait quality roughly tenfold. It is now recorded against the frame at capture time, across all four edges.

## 4.7 An attempted fix that was measured and reverted

Crop selection prefers the largest crops as a proxy for the clearest. For a person walking past a camera, largest means closest, which means side-on. Measured on one clip:

| Crops | Count | Median yaw | Best face similarity |
| --- | --- | --- | --- |
| Selected for embedding | 16 | 78.8° | 0.515 |
| Discarded | 48 | 51.4° | 0.622 |

Since the face branch scores zero past 65°, over half the crops handed to the expensive model were guaranteed to score nothing, while better frontal frames went unused. A fix that spread selection across the track improved median yaw to 61.5° but made results worse — the fused score for that clip fell from 0.359 to 0.006, because frontal frames are further away and therefore smaller, so what was gained in frontality was lost in resolution and frames fell below the minimum quality gate. The change was reverted. The observation stands; that particular remedy does not.

# 5. Evaluation methodology

Two measurement tools were built during this cycle, because the project had been reasoning from assumed constants where it should have been measuring.

## 5.1 Calibration measurement

The shipped appearance anchors (stranger 0.726, genuine 0.881) were measured on "one photograph, two people, no variation in pose or lighting", as the configuration file itself records. A script now derives them from real footage, using the face branch to label which track is whom and leaving ambiguous tracks unlabelled.

genuine  appearance:  min 0.493   median 0.517   max 0.814

impostor appearance:  min 0.516   median 0.567   max 0.710

2 of 3 genuine pairs fall BELOW the shipped impostor anchor

and therefore calibrate to exactly 0.0

The script refuses to suggest replacement anchors here, and says why. The genuine median (0.517) is not above the impostor 90th percentile (0.703). No pair of anchors separates distributions that are not separated. Moving them until the numbers looked better would relabel noise as confidence, so the tool exits with a distinct status and tells the operator to distrust the modality on that footage.

## 5.2 Camera placement, measured

Face quality penalises yaw (turning left or right) and ignores pitch (a camera mounted high, looking down). Whether that omission is safe was tested rather than assumed:

| Camera tilt below horizontal | Faces measured | Median similarity |
| --- | --- | --- |
| 0–5° (eye level) | 41 | 0.732 |
| 5–10° | 7 | 0.863 |
| 10–15° | 2 | 0.784 |
| 15–20° | 2 | 0.733 |
| 20–40° | 10 | 0.745 |

Recognition is flat with respect to downward tilt out to 40°, which is why standard CCTV mounting is high and angled down. Turning sideways is the destructive axis: the same subject scores 0.2-0.5 in profile against 0.75-0.85 head-on. The practical rule is to mount the camera above head height, tilted 15-30° down, pointed along the direction people walk so they approach the lens.

Deliberately not fixed. Because pitch does not enter the quality score, a camera mounted almost vertically overhead would report high confidence while recognition quietly degraded — the same class of failure as section 4.2. No penalty curve was added, because no footage steeper than 40° was available to calibrate one against, and an invented curve would be worse than a documented limitation.

## 5.3 Enrolment angles

Enrolment keeps the top 40% of frames by quality and averages them into a single vector, so it discards profile frames by design. Whether that is right for an angled camera was tested by building templates from different source material and scoring each against probes bucketed by how far the head is turned:

| Enrolled from | 0–20° | 20–40° | 40–65° | 65–90° |
| --- | --- | --- | --- | --- |
| Frontal stills only | 0.778 | 0.471 | 0.432 | 0.503 |
| Angled stills only | 0.683 | 0.513 | 0.559 | 0.558 |
| Frontal + angled stills | 0.773 | 0.519 | 0.520 | 0.567 |
| Head-turn video (shipping) | 0.851 | 0.530 | 0.507 | 0.585 |

A slow head-turn video beats every hand-built combination of fixed poses, because the encoder sees every angle in between rather than two extremes. Past 65° nothing helps: all four templates land at 0.50-0.59, which is the practical wall. Because the system stores one averaged template per person, it cannot be optimal at every angle at once — a limitation discussed in section 8.

# 6. Safeguards and data handling

The system points at people and calls them suspects. The following are implemented, not aspirational.

| Safeguard | Implementation |
| --- | --- |
| No autonomous action | Every match is a pending decision. Alerts fire only on human-confirmed decisions, and sending additionally requires an explicit flag. |
| Reasoning is shown | The console displays per-modality scores and weights, and warns explicitly when a match rests mainly on clothing. |
| Complete audit trail | Append-only log of every enrolment, edit, erasure, match and review, with the operator and the reasoning attached. |
| Encryption at rest | Biometric templates are encrypted with a configured key. Without one, enrolment is refused outright rather than silently falling back to plaintext. |
| Right to erasure | Biometrics can be destroyed irreversibly while keeping the record and its decisions reviewable — what an erasure request actually requires. |
| Deletion is bounded | Records named in a match decision cannot be deleted outright; the request is refused with an explanation rather than orphaning the review queue. |
| Footage is not retained | Uploads are deleted as soon as embeddings are extracted; stale staging directories are swept at startup. |
| Biometrics never committed | Enrolment footage, templates, the database and model weights are excluded from version control and verified before every commit. |

Prototype mode. The --demo flag disables sign-in and waives the encryption requirement so the system can be shown without setup. It prints a warning on every start stating that anyone reaching the port can act, that decisions are recorded against a shared operator, and that it must not be used with real data.

# 7. Technology

| Layer | Technology | Notes |
| --- | --- | --- |
| Detection | YOLOv8n (Ultralytics) | Person class only |
| Tracking | deep-sort-realtime | MobileNet association embedder |
| Face | InsightFace buffalo_l | ArcFace, ONNX Runtime |
| Gait | YOLOv8n-seg + GEI | Classical descriptor; no unlicensed weights |
| Appearance | OSNet x1_0, MSMT17 | Vendored (MIT); re-ID weights, not ImageNet |
| Backend | FastAPI, SQLAlchemy 2.0 | Background jobs for enrol and scan |
| Database | SQLite (Postgres-ready) | Templates, decisions, append-only audit |
| Frontend | React 18, Vite 5 | Monochrome review console |
| Config | pydantic-settings | CLI > environment > YAML |
| Testing | pytest | 458 passing, 6,910 lines of test code |

Development hardware: NVIDIA RTX 3050 (4 GB). The 4 GB limit is the constraint behind section 4.4 and behind the decision to keep face inference on CPU.

# 8. Limitations

Stated plainly, because a system of this kind is more dangerous when its limits are unclear than when its accuracy is imperfect.

### Appearance does not survive a change of clothes

Five of six current identifications rest on appearance, measured within one session with the subject wearing the same outfit as the enrolment video. Across outfits, genuine and impostor distributions overlap almost entirely (section 5.1). This is inherent to what re-identification is, not a defect, and the three-day trust half-life partially mitigates it.

### Gait is enrolled but not yet operational

Gait comparison requires at least three enrolled gait references, because every gait descriptor resembles a generic human outline and the population mean must be removed before comparison. With one reference the system refuses to compare rather than returning a misleadingly high similarity. Two further enrolments would activate it.

### Gait may not survive live scanning

Enrolment retains 600 frames per track; live scanning retains 64, roughly two seconds. In the most recent scan the gait branch produced no probe on either clip examined. Whether this is the buffer length or the specific footage has not been determined and is not asserted either way.

### Profile and heavily disguised subjects

Beyond roughly 65° of head rotation the face branch contributes nothing, and no enrolment strategy tested recovers it. A subject who is both turned away and covered is identifiable only by appearance until gait is operational.

### Single template per person

All enrolment frames are averaged into one vector per modality, so a template cannot be simultaneously optimal for frontal and strongly angled views. Per-pose templates would address this and represent a genuine design change rather than a tuning adjustment.

### Small evaluation set

Results rest on a small number of subjects and clips. Reported figures are measurements, not statistically established performance, and no TAR@FAR or ROC figure is quoted because the sample cannot support one.

# 9. Future work

| Priority | Work | Rationale |
| --- | --- | --- |
| High | Enrol two more subjects with walking video | Activates gait, and provides the multi-subject data every other evaluation needs |
| High | Impersonator-aware evaluation | Separate lookalikes from random strangers when reporting accuracy; per DFW Protocol-1, impersonation is where systems fail at strict operating points |
| Medium | Determine whether gait works during scanning | Measure whether the 64-frame live buffer can support the periodicity gate |
| Medium | Re-derive appearance anchors on a wider set | The tooling exists; it currently refuses because the distributions overlap |
| Medium | Per-region identity weights | The script exists; it needs a multi-identity face set to produce trustworthy weights |
| Lower | Per-pose face templates | Removes the single-averaged-template limitation for mixed camera angles |
| Lower | Evaluate against DFW | The only way to make a defensible claim about real disguise robustness |

# Appendix A. Development history

Thirty-seven commits, most recent first.

| Commit | Date | Description |
| --- | --- | --- |
| acd5e0a | 2026-09-02 | Give gait the walking clip, and stop re-ID exhausting the GPU |
| dca74e4 | 2026-09-01 | Measure re-ID calibration anchors instead of assuming them |
| fb6a561 | 2026-08-25 | Make the face branch notice when the face is covered |
| da3f55e | 2026-08-24 | Show the whole face on a watchlist card |
| cc587fb | 2026-08-24 | Let the watchlist be edited, retired, erased and deleted |
| 4ead74a | 2026-08-24 | Stop re-ID vetoing correct matches, and let gait enrol at all |
| 5c4addc | 2026-08-24 | Fail on an occupied port before printing a success banner |
| 60659b9 | 2026-08-24 | Refuse to start when the pipeline’s lazy imports are missing |
| 3875ce7 | 2026-08-24 | --demo also waives the encryption-key requirement |
| 6285be3 | 2026-08-24 | Fix the startup warning serve.py never updated after SEC-06 |
| 1ff1bbe | 2026-08-23 | Add --demo, which turns sign-in off for showing the prototype |
| 1edde32 | 2026-08-23 | Show the camera preview, and let it take photos |
| c106af0 | 2026-08-23 | Offer one video or separate photos, and fix the broken dropzone |
| 788a2b6 | 2026-08-23 | Move the console to port 4173 everywhere |
| 8715016 | 2026-08-23 | Ask for the footage the models can actually use |
| f16e2bb | 2026-08-23 | Rebuild the console around register, records and identify |
| 805ee8e | 2026-08-23 | Take the preview off the event loop and record audit status |
| cde946f | 2026-08-23 | Show the reviewer the person they are identifying (DES-02) |
| e587356 | 2026-08-23 | Fix four smaller findings: uploads, fusion record, poll loop |
| 225c9af | 2026-08-23 | Fix four logic findings: cadence, writer fps, alert replay, queue |
| abc98dc | 2026-08-23 | Load re-ID weights in the re-ID branch (DES-01) |
| 3285e93 | 2026-08-23 | Refuse to write biometric templates unencrypted (SEC-06) |
| 16c2bf9 | 2026-08-23 | Require an account to use the console (SEC-01) |
| 8723d0a | 2026-08-22 | Fix the critical and high audit findings, first pass |
| 6c6cd48 | 2026-08-22 | Add the security and correctness review of c80518f |
| c80518f | 2026-08-20 | Add the upload workflow: enrol and search footage from the browser |
| a989974 | 2026-08-20 | Add alerting, the evaluation harness and the extensions (9-11) |
| c11065a | 2026-08-20 | Add the React review console with an explainability view (8) |
| cdce75b | 2026-08-20 | Add the API and database with an audited human-confirm workflow (7) |
| 07410f4 | 2026-08-20 | Add the keyless attention head, trained only on synthetic data (6) |
| 39da123 | 2026-08-20 | Add baseline fusion and fix gait’s discrimination (phase 5) |
| 339e17c | 2026-08-20 | Add the re-ID branch: OSNet appearance embeddings (phase 4) |
| 3d488dc | 2026-08-20 | Add the gait branch: silhouettes, gait cycles, GEI (phase 3) |
| 9539cf1 | 2026-08-20 | Add the face branch: enroll a person, match by ArcFace (phase 2) |
| 73c0982 | 2026-08-19 | Fix issues found running the phase-1 pipeline end to end |
| a8615d9 | 2026-08-19 | Scaffold repo and add YOLOv8 + DeepSORT pipeline (phases 0-1) |

# Appendix B. Key configuration

| Setting | Value | Meaning |
| --- | --- | --- |
| fusion.threshold | 0.55 | Score at which a candidate is surfaced for review |
| fusion.strategy | quality_weighted | Weighted simultaneous fusion, not a fallback chain |
| face.max_yaw_deg | 65 | Head rotation past which face scores zero |
| face.ideal_face_height | 112 | Pixels of face height for full resolution credit |
| face.occlusion_aware | true | Visibility factor added this cycle |
| gait.min_periodicity | 0.55 | Rhythm required to accept a walk |
| gait.max_area_cv | 0.10 | Silhouette stability required |
| gait_min_references_for_centring | 3 | Gait references needed before comparison |
| reid.trust_half_life_days | 3.0 | Appearance staleness decay |
| reid.max_batch | 64 | Crops per GPU forward pass, added this cycle |